  - STAND ALONE LOCKSETS

    - Battery Powered Cylindrical Lockset with Digital Keypad and Proximity card reader:

      - Product: Model E75P - Entrycheck Standalone Lockset with Digital Keypad and/or Prox Card Reader as manufactured by SDC Security Door Controls.

        - Battery powered electronic lockset for 3000 users. Standard features include an ANSI Grade 1 cylindrical lockset with a lever clutch mechanism, 4-6 digit pin code, all weather keypad and housing, keypad programming, 32 user groups, multi-year scheduling, temporary users multiple zones, real time clock, auto daylight savings adjustment and more. 9/16 inch (9.5 mm) latch, 4-7/8 inch (124 mm) strike standard.

        - UL Listed, USA and Canada for 3 hour "A" label fire rated doors. ANSI A156.25, Electrified Lockset Compliant. ANSI A156.2, Grade 2 Compliant.

        - ANSI A117.1 Accessibility Code Compliant.

        - ADA Compliant. California State Reference Code (Formerly Title 19, California State Fire Marshal):  Compliant. Levers Galaxy and Eclipse return to within 1/2 inch (13mm) of the door face.

** NOTE TO SPECIFIER **  Lever Handles with/without cylinders. Delete lever not required.

      - G1 - Galaxy lever, cylinder with keyway as required.

      - E1 - Eclipse lever, cylinder with keyway as required.

      - G5 Galaxy lever IC core prep , Tailpiece required.

      - E5 Eclipse lever IC core prep , Tailpiece required.

** NOTE TO SPECIFIER **  Black Dull Chrome is standard. Delete finish for Keypad and Lock that are not required.

      - Keypad Finish:

        - Q - Dull chrome

        - B – Black

      - Lock Finish:

        - Q - Dull chrome

        - C – Polished Brass

        - P - Bright Chrome

** NOTE TO SPECIFIER **  Delete cards and key fobs not required.

      - Card: HID1326-100 ProxCard II (100 cards) as manufactured by SDC Security Door Controls.

      - Card: HID1326-25 ProxCard II (25 cards) as manufactured by SDC Security Door Controls.

      - Key Fob: HID1346-10 ProxKey II (10 key fobs) as manufactured by SDC Security Door Controls.

      - Key Fob: HID1346-100 ProxKey II (100 key fobs) as manufactured by SDC Security Door Controls.

    - Battery Powered Cylindrical Lockset with Digital Keypad and Proximity card reader with Bluetooth enabled Software

      - Product: Model E75PS - Entrycheck Standalone Lockset with Digital Keypad and Prox Card Reader and Bluetooth software as manufactured by SDC Security Door Controls.

        - Battery powered electronic lockset for 3000 users. Standard features include an ANSI Grade 1 cylindrical lockset with a lever clutch mechanism, 4-6 digit pin code, all weather keypad and housing, keypad programming, 32 user groups, multi-year scheduling, temporary users multiple zones, real time clock, auto daylight savings adjustment and more. 9/16 inch (9.5 mm) latch, 4-7/8 inch (124 mm) strike standard.

        - UL Listed, USA and Canada for 3 hour "A" label fire rated doors. ANSI A156.25, Electrified Lockset Compliant. ANSI A156.2, Grade 2 Compliant.

        - ANSI A117.1 Accessibility Code Compliant.

        - ADA Compliant. California State Reference Code (Formerly Title 19, California State Fire Marshal):  Compliant. Levers Galaxy and Eclipse return to within 1/2 inch (13mm) of the door face.

        - Point and click Microsoft PC based software that supports up to 1000 unique doors and 3000 user credentials. Manages door access parameters, access schedules, User Groups, Holidays and temporary users.

        - Lock settings are easily cloned to additional lock sets and lock data with User data may easily be imported from existing databases.

        - Bluetooth wireless information transfer of lock programming and Audit Trail may be uploaded and downloaded to a Bluetooth enabled PC.

        - Audit Trail information is stored for up to 10,000 events with Users logged with time and date. Remote activation and Door alarm activities are also logged. Audit Trail information can easily be exported to Excel spreadsheets for sorting or printing.

        - PC Requirements: Windows 7 or Windows 8. Processor speed of 2 GHz or faster with Hard disk space of 1 GB and 2 GB RAM recommended.

** NOTE TO SPECIFIER **  Lever Handles with/without cylinders. Delete lever not required.

      - G1 - Galaxy lever, cylinder with keyway as required.

      - E1 - Eclipse lever, cylinder with keyway as required.

      - G5 Galaxy lever IC core prep , Tailpiece required.

      - E5 Eclipse lever IC core prep , Tailpiece required.

** NOTE TO SPECIFIER **  Black Dull Chrome is standard. Delete finish for Keypad and Lock that are not required.

      - Keypad Finish:

        - Q - Dull chrome

        - B – Black

      - Lock Finish:

        - Q - Dull chrome

        - C – Polished Brass

        - P - Bright Chrome

** NOTE TO SPECIFIER **  Delete cards and key fobs not required.

      - Card: HID1326-100 ProxCard II (100 cards) as manufactured by SDC Security Door Controls.

      - Card: HID1326-25 ProxCard II (25 cards) as manufactured by SDC Security Door Controls.

      - Key Fob: HID1346-10 ProxKey II (10 key fobs) as manufactured by SDC Security Door Controls.

      - Key Fob: HID1346-100 ProxKey II (100 key fobs) as manufactured by SDC Security Door Controls.