  - ELECTRIC BOLT LOCKS

** NOTE TO SPECIFIER **  The Spacesaver®, first designed and patented by SDC, is a fundamental innovation in electric locking technology for access control applications. The stainless steel bolt projects at right angles to the lock mechanism, allowing installation of Spacesaver® locks, by means of a simple cutout, in virtually any standard 1.5 inches frame. , or in most door lock stiles. Delete if not required.

      - Product: Model Spacesaver 1190AU - Failsafe Bolt Lock, Narrow, Extra Heavy Duty as manufactured by SDC Security Door Controls. 0.7Amp @ 24VDC. Standard finish type U.

      - Product: Model Spacesaver 2090AU - Failsecure Bolt Lock, Narrow, Extra Heavy Duty as manufactured by SDC Security Door Controls. 2.5 Amp In rush, 0.4 Amp seated @ 24VDC. Standard finish type U. Note – Will not unlock without power.

** NOTE TO SPECIFIER **  Conventional failsafe locking bolts rely solely on a spring to retract the bolt while the FS23M bolt will unlock with or without the assistance of the spring. This is the only electric bolt lock listed by the California State Fire Marshal for locking of perimeter entrance and emergency exit doors after-hours.

** NOTE TO SPECIFIER **  Delete option not required.

      - PR-1000 – Power regulator and heat reducer for 1091, 1291, 1190, FS23

      - Accessory: B - Magnetic Bolt Status Switch. SPDT 0.25 Amp @ 30VDC. Indicates bolt locked and unlocked.

      - Accessory: D - Mechanical Door Position Switch. SPDT 5 Amps at 30VDC. Integral to the auto relock ball switch assembly. Indicates, door closed and door open. Recommended for minimum security applications only.

      - Accessory: BR64XL - Rectifier for AC operation.