  - MAGNETIC SHEAR LOCKS

    - Electromagnetic Shear Lock:

      - The electromagnetic lock shall incorporates a "floating" armature assembly and hardened alloy steel locking "tabs" on both the lock and armature assemblies, that allow adjustment both vertically and laterally to compensate for wide door gaps and warped or misaligned doors.

      - Static sensor activates lock only when door is aligned and at rest.

      - Automatic sensing 12/24 VDC input.

** NOTE TO SPECIFIER **  Delete model not required. Finish not required for 1561, 1562, 1565, 1566

      - Product: Concealed 2000 lb (907 Kg) Holding Force with Integrated Electronics: Model 1561 Hi/Shear EMLock as manufactured by SDC Security Door Controls.

      - Product: Concealed 2000 lb (907 Kg) Holding Force with External Electronics: Model 1562 Hi/Shear EMLock as manufactured by SDC Security Door Controls.

      - Product: Concealed 2700 lb (1225 Kg) Holding Force with Integrated Electronics: Model 1565 Hi/Shear EMLock as manufactured by SDC Security Door Controls.

      - Product: Concealed 2700 lb (1225 Kg) Holding Force with External Electronics: Model 1566 Hi/Shear EMLock as manufactured by SDC Security Door Controls.

      - Product: Surface Mount 2000 lb (907 Kg) Holding Force with Integrated Electronics: Model 1561S Hi/Shear EMLock as manufactured by SDC Security Door Controls.

      - Product: Surface Top Jamb Mount 2000 lb (907 Kg) Holding Force with Integrated Electronics: Model 1561TJ Hi/Shear EMLock for in-swing doors as manufactured by SDC Security Door Controls.

      - Product: Semi-Concealed 2000 lb (907 Kg) Holding Force with External Electronics: Model  1562SC Hi/Shear EMLock as manufactured by SDC Security Door Controls.

      - Product: Semi-Concealed 2000 lb (907 Kg) Holding Force for Herculite doors with External Electronics: Model 1562SCHDB Hi/Shear EMLock as manufactured by SDC Security Door Controls.

        - ½” glass (13mm)

        - 3/4” glass (19mm)

      - Provide armature and mounting kit as indicated or required for frame material and profile. Verify push or pull side if not indicated. Provide installation on secure side of opening (typical).

** NOTE TO SPECIFIER **  Finish V is standard for 1561S, 1561TJ, 1562SC and 1562SCHDB. Delete finish not required. Plated finishes are special order.

      - Finish: V - 628 Aluminum.

      - Finish: X - Dark Bronze powder coat (313/613 equal).

      - Finish: Y - 335 Black Anodized.

      - Plated Finish: Q - 626 Dull Chrome.

      - Plated Finish: C - 605 Bright Brass. Powder coat

      - Plated Finish: D - 606 Dull Brass. Powder coat

      - Plated Finish: P - 625 Bright Chrome.

** NOTE TO SPECIFIER **  Optional. Delete accessory not required.

      - Accessory: B - Bond sensor indicates proper armature contact or reduced holding power due to improper armature contact or tampering. SPDT 0.25Amp (not available on 1562/1566).

      - Accessory: D - Door position switch indicates door open or closed. SPDT 0.25 Amp maximum.

      - Accessory: L-3 - External LED status indicator, mounts separately through the frame and may be used to indicate lock is energized or wired to bond sensor for door status.