  - MAGNETIC LOCKS AND GATE LOCKS

    - Electromagnetic Gate Lock:

** NOTE TO SPECIFIER **  Delete model not required.

      - Product: Model 1575 - Single EMLock with magnetic bond and door status sensors.

      - Product: Model 1576 - Single EMLock face drilled mounting with magnetic bond and door status sensors.

        - Case shall be totally sealed in an epoxy filled stainless steel case. A threaded conduit fitting shall ensure weather resistant protection of the wiring.

        - Holding Force 1200 lb (544 Kg), Voltage Input: 450 mA at 12 VDC, 225 mA at 24 VDC. Bond Sensor Rating: 125 mA at 12/24 VDC.

** NOTE TO SPECIFIER **  Delete power source requirements not required.

      - 12 VDC and 24 VDC.

** NOTE TO SPECIFIER **  Delete accessory not required.

      - Accessories: 1576-BK: Mounting bracket for 1576.

      - Accessories: 1576-AB: Armature mounting bracket for 1576.

** NOTE TO SPECIFIER **  Delete accessory not required. For swinging gate or top jamb door application

      - Accessories: 1576-MP: Lock mounting plate for 1576.

      - Accessories: 1576-ZB: Lock mounting Z bracket for 1576 (includes 1576-MP).