  - DELAYED EGRESS LOCKS

    - Mini Delayed Egress Lock, Integrated:

      - 650 lb (748 Kg) holding force exit delay and nuisance delay per code specification, subdued alarm, two tones Code compliant Manual or auto power-up modes.

** NOTE TO SPECIFIER **   Delete model not required. Specify 2 units for double doors

      - Product: Model 1581S - Mini Delayed Egress lock 650lbs. holding force as manufactured by SDC. (Specify 2 units for double doors and select cable)

      - Operation: ND - IBC, IFC, NFPA 101, NFPA 1-UFC, UBC, SBC Compliant.  15 second fixed exit delay, 1 second nuisance delay

      - Operation: NH - IBC, IFC, NFPA 101, NFPA 1 -  UFC, SBC Compliant.  30 second fixed exit delay,  1 second nuisance delay.

      - Operation: NC - IBC, IFC, California Building Code/OSHPD, NFPA 101, NFPA 1- UFC, UBC, SBC Compliant.  15 second fixed exit delay, 1 second nuisance delay, manual power up

      - BD - BOCA National Building Code Compliant.  15 second fixed exit delay, 1 second nuisance delay, auto reset 30 seconds after door closure.

      - BC - BOCA Chicago Code Compliant.  15 second fixed exit delay, 0 second nuisance delay, auto reset 30 seconds after door closure.

      - BH - BOCA National Building Code Compliant.  30 second fixed exit delay, 1 second nuisance delay, auto reset 30 seconds after door closure.

** NOTE TO SPECIFIER **  Finish V is standard. Delete finish not required. Plated finishes are special order.

      - Finish: V - 628 Aluminum.

      - Finish: X - Dark Bronze powder coat (313/613 equal).

      - Finish: Y - 335 Black Anodized.

      - Plated Finish: Q - 626 Dull Chrome.

      - Plated Finish: C - 605 Bright Brass. Powder coat

      - Plated Finish: D - 606 Dull Brass. Powder coat

      - Plated Finish: P - 625 Bright Chrome.

** NOTE TO SPECIFIER **   Delete accessory not required.

      - Accessory: D - Door Position Status. Provides remote monitoring.

      - Accessory: B - Magnetic Bond Alert Sensor.

      - Accessory 1581S-TC3 – 3 ft Tandem cable

      - Accessory 1581S-TC10 – 10 ft Tandem cable