  - ELECTRIC BOLT LOCKS

** NOTE TO SPECIFIER **  The Spacesaver®, first designed and patented by SDC, is a fundamental innovation in electric locking technology for access control applications. The stainless steel bolt projects at right angles to the lock mechanism, allowing installation of Spacesaver® locks, by means of a simple cutout, in virtually any standard 1.5 inches frame. , or in most door lock stiles. Delete if not required.

    - Conventional Bolt Locks, Mortise: Not for Wood applications

** NOTE TO SPECIFIER **  The 110 and 210 are the most basic bolt locks available. The bolt assembly is adjustable forward up to 1 inch (25.4 mm) past the face plate to accommodate wide door gaps or special applications. Delete model and dimensions if not required. Not for wood applications

** NOTE TO SPECIFIER **  The 160 and 260 are equipped with an auto relock switch. Field adjustable for wide door gaps, the auto relock switch keeps the bolt retracted when the door is open and when depressed upon door closure, relocks the door automatically. The bolt assembly is also adjustable for wide door gaps. Delete model and dimensions if not required. Not for wood applications

      - Product: Model 160 - Failsafe Mortised Bolt Lock as manufactured by SDC Security Door Controls. Dual Voltage Coil: 900/450 mA @ 12/24 VDC.

      - Product: Model 260 - Failsecure Mortised Bolt Lock as manufactured by SDC Security Door Controls. Dual Voltage Coil: 900/450 mA @ 12/24 VDC. Note – Will not unlock without power

        - Faceplate 4.875 inches H by 1.25 inches W by 0.125 inch D (126.84 by 31.75 by 3.175 mm).

        - Strike 4 inches H by 1.25 inches W by 0.125 inch D (101.6 by 31.75 by 3.175 mm).

      - Bolt: Solid stainless steel finish 0.625 inch diameter by 0.5 inch throw (15.87 mm by 12.7 mm).

      - Finish: V - 628 Clear Anodized Aluminum.