  - ELECTRIC STRIKES

    - Electric Strikes for Cylindrical Locks:

      - Product: Model 25-4U - 4.875 inches (124 mm), 300mA at 12VDC or 150mA at 24VDC as manufactured by SDC Security Door Controls.

      - For up to 5/8 inch (16 mm) latch bolts.

      - Field selectable failsafe/failsecure.

      - Field selectable 12/24 V AC/DC.

      - Latch status output.

      - Non handed reversible.

      - All stainless steel parts with durable die cast body for corrosion resistance, 630 Satin stainless steel face plate.

      - Not recommended for wood frame applications

      - ** NOTE TO SPECIFIER ** Finish: Delete finish not required.

      - Finish: U - 630 Dull Stainless Steel.

      - Finish: V - 628 Dull Aluminum.

      - Finish: Y - 335 Black Anodized.

      - Finish: D - 606 Dull Brass.

      - Finish: H - 613 Oil Rubbed Bronze.

** NOTE TO SPECIFIER **  Delete Buzzer and Keeper options if not required.

      - Provide Buzzer RMB 12/24 V AC/DC operation.