  - EXIT SWITCHES

    - Vandal Resistant Exit Switches:

** NOTE TO SPECIFIER **  The SDC 460 series 463 is a heavy duty, vandal resistant exit switch designed for harsh indoor or outdoor environments. The stainless steel piezoelectric technology is also ideal for high traffic applications. The switch assembly design ensures superior performance in virtually any environment. Delete if not required.

      - Vandal resistant stainless steel piezoelectric button and faceplate. No moving parts. Billion cycle button technology. Bi-color status indicator, Red and Green. Integrated timer, adjustable 1-30 seconds or latching.  
 0.08 inch (2mm) Stainless Steel U – 630, security screws.

        - Product: Model 463/463N - Integrated  1-30 Second Timer or latching SPDT 3 Amp contact (PUSH TO EXIT / EXIT) as manufactured by SDC Security Door Controls.

        - 463U – Single gang box mounting

        - 463NU – Narrow mullion mounting

        - Input Voltage: 12/24 VDC. Current: LED On - Idle 30mA , Active-150mA.

        - Outputs: SPDT , 3 Amp at 30 VDC.

        - Relay Life Expectancy: 100,000 cycles at 3 Amp, 30 VDC. 200,000 cycles at 1.5 Amp 30 VDC.

        - Operating Temperature: -40 degree F to +160 degree F (-40 degree C to +70 degree C).