  - MISCELLANEOUS

    - Emergency Break Glass Release:

      - SDC 490 series emergency door release stations shall include 2-SPDT contacts for release of all doors on the same circuit and remote annunciation. A built-in local alarm is standard.

      - Product as manufactured by SDC Security Door Controls.

** NOTE TO SPECIFIER **  Delete model not required.

      - Product: Model 491 - Blue break glass station, emergency door release, 2-SPDT 10 Amp contacts with built-in local alarm (12/24V), 2 replacement glass included. Single gang box mounted.

      - Product: Model 491-GL4 - Four replacement glass plates for 491.

      - Product: Model 491-BB - Blue surface mount box for 491.

      - Product: Model 492 - Blue pull station, emergency door release, 2-SPDT 10 Amp contacts with built-in local alarm (12/24V), 2 replacement glass rods included. Single gang box mounted.

      - Product: Model 492-GL4 - Four replacement glass rods for 492.

      - Product: Model 492-BB - Blue surface mount box for 492