  - ELECTRIC STRIKES

    - Extra Heavy Duty Electric Strikes:

      - Multiple faceplates provided to accommodate all cylindrical and mortise locksets without/with dead bolts.

      - UL Listed for fire doors. Fits standard ANSI frame prep with slight modification. All stainless steel parts.

      - Product: Model 55-ABCU- 300mA/600mA at 24VDC/12VDC as manufactured by SDC Security Door Controls. For cylindrical and mortise locksets without deadbolt

      - Product Model 55-AU – For cylindrical locksets

      - Product Model 55-BU –For mortise locksets with deadlatch below latch bolt

      - Product Model 55-CU – For mortise locksets with deadlatch above latchbolt

      - Product Model 55-DU – For mortise locksets with deadbolt and the deadlatch below the latch bolt (Door will not close with deadbolt projected)

      - Product Model 55-EU – For mortise locksets with deadbolt and no deadlatch (Door will not close with deadbolt projected)

      - Product Model 55-FU – For mortise locksets with deadlatch and deadbolt above latchbolt. (Door will not close with deadbolt projected)

** NOTE TO SPECIFIER **  Delete latch, and lock status and deadbolt status if not required.

      - Provide Latch Bolt Status – LBM.

      - Provide Latch & Lock Status – LBM/LCM.

      - DBM-L or DBM-R – Deadbolt monitor switch. Specify –R or -L