  - STAND ALONE LOCKSETS

    - Battery Powered Cylindrical Lockset with Digital Keypad:

      - Product: Model E75K - Entrycheck Standalone Lockset with Digital Keypad as manufactured by SDC Security Door Controls.

        - Battery powered electronic lockset for 3000 users. Standard features include an ANSI Grade 1 cylindrical lockset with a lever clutch mechanism, 4-6 digit pin code, all weather keypad and housing, keypad or computer programming, 32 user groups, multi-year scheduling, temporary users, multiple zones, real time clock, auto daylight savings adjustment and more. 9/16 inch (9.5 mm) latch, 4-7/8 inch (124 mm) strike standard.

        - UL Listed, USA and Canada for 3 hour "A" label fire rated doors. ANSI A156.25, Electrified Lockset Compliant. ANSI A156.2, Grade 2 Compliant.

        - ANSI A117.1 Accessibility Code Compliant. ADA Compliant. California State Reference Code (Formerly Title 19, California State Fire Marshal):  Compliant. Levers Galaxy and Eclipse return to within 1/2 inch (13mm) of the door face.

** NOTE TO SPECIFIER **  Lever Handles with/without cylinders. Delete lever not required.

      - Lever and Cylinder: G1 - Galaxy lever, cylinder with keyway as required.

      - Lever and Cylinder: E1- Eclipse lever, cylinder with keyway as required.

      - G5 - Galaxy lever IC core prep, Tailpiece required.

      - E5 Eclipse lever IC core prep, Tailpiece required.

** NOTE TO SPECIFIER **  Black Dull Chrome is standard. Delete finish for keypad and lock that are not required.

      - Keypad Finish:

                - Q - Dull chrome

                - B – Black

                - C – Polished Brass

                - P – Bright Chrome

      - Lock Finish:

        - Q- Dull chrome

        - C – Polished Brass

        - P – Bright Chrome