  - KEY SWITCHES

    - Key Switches:

** NOTE TO SPECIFIER **  Delete faceplate not required.

      - Single gang, wall mounted, anti-tamper recessed key cylinder:

        - Product: Model 7XX (XX= Function 01 -13 below) -  20 ga. stainless steel. 2.875 inches by 4.5 inches by 20 Ga. (73 by 114 by 0.912 mm) as manufactured by SDC Security Door Controls.

      - Narrow 1.75 inches wide, frame mounted:

        - Product: Model 7XXN (XX= Function 01 -13 below)   - 20 ga. stainless steel.1.75 inches by 4.5 inches by 20 ga. (45 by 114 by 0.912 mm) as manufactured by SDC Security Door Controls.

      - Two gang faceplate provides for the use of a larger junction box to accommodate time delays, audible annunciation and increased wiring.

        - Product: Model 7XXT(XX= Function 01 -13 below)  - 20 ga. stainless steel.4.5 inches by 4.5 inches by 20 Ga. (112 by 114 by 0.912 mm) as manufactured by SDC Security Door Controls.

      - Single gang, wall mounted:

        - Product: Model 8XX (XX= Function 01 -13 below)  - 0.25 inch (6 mm) anodized aluminum. 3.25 inches by 5 inches by .25 inch D (83 by 127 by 6.35 mm) as manufactured by SDC Security Door Controls.

      - Narrow, 1.75 inches wide frame mounted. Four mounting screws.

        - Product: Model  8XXN (XX= Function 01 -13 below) - 0.25 inch (6 mm) anodized aluminum. 1.75 inches by 5 inches by .25 inches D (45 by 127x 6.35 mm) as manufactured by SDC Security Door Controls.

** NOTE TO SPECIFIER **  Functions available in both series and sizes unless noted. Delete function not required.

      - Function 01: Alternate action SPDT. Turning the key left or right actuates and latches the contact. Contact position is maintained until the key is inserted and turned again.

      - Function 02: Momentary SPDT. Turning the key left or right momentarily actuates the contact.

** NOTE TO SPECIFIER **  704 and 804AL only. Delete if not required.

      - Function 04: Momentary SPDT plus 10TD Electronic Mini Timer SPDT 2 Amp at 30 VDC, adjustable 1 - 60 seconds. Turning the key left or right activates an electronically timed contact for 1 - 60 seconds.

      - Function 05: Alternate action DPDT. Turning the key left or right actuates and latches the contact. Contact position is maintained until the key is inserted and turned again.

      - Function 06: Momentary DPDT. Turning the key left or right momentarily actuates the contacts.

      - Function 07:  #1 Momentary SPDT - Turning the key left momentarily actuates contact #1. #2 Momentary SPDT - Turning the key right momentarily actuates contact #2.

      - Function 08: #1 Momentary SPDT - Turning the key in one direction momentarily actuates contact #1. #2 Alternate action SPDT - Turning the key in opposite direction actuates and latches contact #2 until the key is inserted and turned again.

      - Function 09:  #1 Alternate action SPDT - Turning the key left actuates and latches contact #1. #2 Alternate Action SPDT - Turning the key right actuates and latches contact #2.

** NOTE TO SPECIFIER **  710T only. Delete if not required.

      - Function 10: #1 Momentary SPDT plus 10TD Electronic Mini Timer, SPDT 2Amp, 1-60 second - Turning the key in one direction activates an electronically timed contact for 1 - 60 seconds. #2 Alternate action SPDT - Turning the key in opposite direction actuates and latches contact #2.

      - Function 11: #1 Momentary DPDT - Turning the key left momentarily actuates contact #1. #2 Momentary DPDT - Turning the key right momentarily actuates contact #2.

      - Function 12: #1 Momentary DPDT - Turning the key in one direction momentarily actuates contact #1. #2 Alternate action DPDT - Turning the key opposite direction actuates and latches contact #2 until the key is inserted and turned again.

      - Function 13: #1 Alternate action DPDT - Turning the key left actuates and latches contact #1. #2 alternate action DPDT - Turning the key right actuates and latches contact #2.

** NOTE TO SPECIFIER **  Finish options for 700 series only. Finish: U 630 standard. Delete finish not required. 800 series is only 628 – anodized aluminum

      - Finish: U 630 - Dull stainless steel.

      - Finish: C 605 - Bright brass.

      - Finish: D 606 - Dull brass.

      - Finish: H 613 - Oil rubbed bronze.

      - Finish: P 625 - Bright Chrome.

** NOTE TO SPECIFIER **  Delete options not required. L1R, L1G, L2, LT select only one

      - Accessory: ATS - Anti-Tamper switch.

      - Accessory: L1 - One LED; specify red or green.

      - Accessory: L2 - Two LED's; red and green.

      - Accessory: LT - One tri-color LED; red green and amber.

      - Accessory: 10TD - Mini timer.

      - Accessory: CYL-KD - Key cylinder with two keys.

** NOTE TO SPECIFIER **  Available with 700T only.

      - Accessory: RMB - Buzzer. May be wired to sound when door is unlocked. 12 or 24VDC operation.

** NOTE TO SPECIFIER **  Available with 700T only.

      - Accessory: SN - 85 db siren, 3-28 VDC.