  - EXIT SENSE BARS AND SWITCH BARS

** NOTE TO SPECIFIER **  Non-Latching Push Bar for: Magnetic Lock Release; Access Control Request-to-Exit; Delayed Egress Trigger; Alarm Shunt; CCTV or Alarm Activation. Delete if not required.

    - Exit Sense Bar:

** NOTE TO SPECIFIER **  Delete model not required.

      - Product: Model PSB560 - Pressure Sensitive Exit Bar as manufactured by SDC Security Door Controls.

      - Activation Force: Factory Set: 5 lb (2.2 Kg). Field adjustable to 15 lb (6.8 Kg).

      - Armored control wire loop PT-2U included

      - Height: 2.375 inches (60.3 mm).

      - Depth: 1.875 inches (47.6 mm).

      - Voltage Input: 12/24 VDC Voltage sensing. No field adjustment.

      - Current: 20 mA at rest, 115 mA active.

      - Operating Temp: 0 degree F to 150 degree F (-18 degree C to 65 degree C).

** NOTE TO SPECIFIER **  One contact standard for MSB550 and two contacts optional. Two contacts standard for PSB560. Delete contact requirement not required.

      - Output: Two SPDT Dry contacts.

** NOTE TO SPECIFIER **  36 inches (914 mm) Standard. May be field cut to length. Delete length not required.

      - Width: 36 inches (914 mm).

      - Width: 42 inches (1,067mm).

      - Width: 48 inches (1,219 mm).

** NOTE TO SPECIFIER **  Options: Delete option not required.

      - Accessory: 500GL-KIT - Glass mounting kit for full glass and herculite doors with stiles. 2 mounting blocks and adhesive for 8 doors.

      - Accessory: 500GL-MB - Additional mounting blocks (2).

      - Accessory: 500-SK - Shim kit used to raise bar from door surface. Recommended for wood or hollow metal doors.

** NOTE TO SPECIFIER **  Finish: V -  standard for PSB/MSB. Finish: X available on MSB only. Delete finish not required.

      - Finish: V - 628 Aluminum anodized.

      - Finish: S - 629 Bright stainless steel.

      - Finish: U - 630 Brushed stainless steel.

      - Finish: C - 605 Bright brass.

      - Finish: Y - 335 Black anodized.