  - ELECTRIFIED EXIT DEVICE (Check with factory when multiple electrified options are required for an exit bar)

    - Exit Devices:

      - Product: Spectra S6000 Series Exit Device as manufactured by SDC Security Door Controls

      - Product: Spectra N6000 Series Narrow Head Exit Device as manufactured by SDC Security Door Controls

      - UL & ULC Listed panic and fire exit hardware. ANSI/BHMA A156.3-2001 Grade 1 Compliant. CA Title 19 Compliant. Meets/Exceeds ADA requirements. UBC 97, Standard 7.2.

** NOTE TO SPECIFIER **  Model Delete model not required.

      - Model S61 - Rim or rim mount.

      - Model N61 – Rim or Rim mount - Narrow

      - Model S62 - Surface vertical rod.

      - Model N63 – Surface vertical rod - Narrow

      - Model S63 -  Mortise.

      - Model S68 - Concealed vertical rod for metal doors.

      - Model N68 – Concealed vertical rod - Narrow

** NOTE TO SPECIFIER **  ANSI Functions: Delete function not required.

      - Function: 01 - Exit only, No Trim.

      - Function: 02 - Dummy Pull or Lever (opens when dogged).

      - Function: 03 - Pull with cylinder or cylinder only, key latch retraction.

      - Function: 08 - Lever with key locks/unlocks lever.

      - Function: 14 - Lever trim always active.

** NOTE TO SPECIFIER **  Panic or Fire Rated. Delete dogging not required.

      - Dogging: P - Panic Exit Device, mechanical dogging.

      - Dogging: P CD - Panic Exit Device, key cylinder dogging. (Mortise Cyl not included)

      - Dogging: F - Fire Exit Device, dogging not permitted.

** NOTE TO SPECIFIER **  Delete handing not required.

      - Handing: RR - Right hand reverse bevel, RHRB.

      - Handing: LR - Left hand reverse bevel, LHRB.

** NOTE TO SPECIFIER **  Finish: Delete finish not required.

      - Finish: U - 630 Dull Stainless Steel. (Std)

      - Finish: C - 605 Bright Brass.

      - Finish: H - 613 Oil Rubbed Bronze.

      - Finish: D - 606 Dull Brass.

      - Finish: P - 625 Bright Chrome.

      - Finish: G - 612 Dull Bronze.

      - Finish: Q - 626 Dull Chrome.

** NOTE TO SPECIFIER **  Length: Delete door width not required.

      - Length: 36 inches (914 mm) door.

      - Length: 42 inches (1067 mm) door.

      - Length: 48 inches (1219 mm) door.

** NOTE TO SPECIFIER **  Delete signal output not required.

      - Signal Output: R - Request-To-Exit Output, SPDT.

** NOTE TO SPECIFIER **  Delete if not required. Not available with other options – E,G, Alarm A/B

      - Delayed Egress: Model DE - Electrification of exit device and wall mount controller with keypad control and reset.

** NOTE TO SPECIFIER **  Delete accessory not required.

        - Provide DEC-J - 3 Gang Interior surface mount box for 101-DE.

        - Provide SHD-J - Shroud for surface mount box DE-J outdoor use.

** NOTE TO SPECIFIER **  Delete if not required. Not available with other options – E,G, Alarm A/B

      - Delayed Egress: Model KDE - Electrification of exit device and wall mount controller with both keypad and key switch control and reset, less cylinder.

      - DES – Delayed Egress bar for pair of doors. Additional exit device equipped with delayed egress mechanism, less controller. Must be ordered together with DE and device.

      - KDES – Delayed Egress bar for pair of doors, additional exit device equipped with delayed egress mechanism, less controller. Must be ordered with KDE and device.

** NOTE TO SPECIFIER **  Exit alarm kit for rim mount, mortise, vertical rod. Delete if not required. Not available with other options – E, G, DE/KDE

** NOTE TO SPECIFIER **  Trim: Delete trim not required.

      - Mechanical Trim: EK - Eclipse (Function and finish to match device).

      - Mechanical Trim: GK - Galaxy (Function and finish to match device).

      - Mechanical Trim: SP - Saturn (Function and finish to match device).

      - Mechanical Trim EN – Eclipse Narrow (Function and finish to match device).

      - Mechanical Trim GN – Galaxy Narrow (Function and finish to match device)

      - Mechanical Trim SN – Saturn Narrow (Function and finish to match device)

** NOTE TO SPECIFIER **  Electric Trim: Delete trim not required.

      - Electrified Trim: EKE03 - Eclipse (finish to match device).

      - Electrified Trim: EKE03_LT - Eclipse with LED (finish to match device).

      - Electrified Trim: GKE03 - Galaxy (finish to match device).

      - Electrified Trim: GKE03_LT -  Galaxy with LED (finish to match device).

        - Failsecure, field reversible to failsafe, audible tone, 4 ft cable and connectors, 12/24 VDC at 600/300 mA.

** NOTE TO SPECIFIER **  Delete Electric Trim Options not required.

        - LT - Tricolor LED Indicator.

        - LS - Locked/unlocked status, SPDT 3 Amp contact.