  - ELECTRIFIED CYLINDRICAL LOCKSETS

    - Electrified SDC Cylindrical Lock:

      - Product: Model Z7200 Series Electra Pro Electrified Cylindrical Lockset as manufactured by SDC Security Door Controls. Exceeds ANSI A156.2 Grade 1 specifications.

      - Request-to-Exit (REX) Output Standard. Turning the inside lever for egress activates the built in REX output providing a momentary signal to the access control REX input for alarm shunt during egress. Outside locked, unlocked by an access control or key from the outside.

        - Operation: Locked on exterior only. SDC electrified Z7200 series.

        - Voltage: Dual 12/24 VDC Input.

        - Current: 600/300 mA at 12/24 VDC.

        - Request to Exit (REX): Output SPDT 3 Amps at 30 VDC.

        - Backset and Latch Bolt: 0.56 inch (14 mm) throw with 2.75 inches (70 mm) backset.

        - Strike: ST1 - ANSI A115.2, 1-1/4 inches by 4-7/8 inches (32 mm by 124 mm).

        - Lever clutch for vandal resistance.

        - Door Thickness: 1.625 inches to 2.125 inches (41 mm to 54 mm).

** NOTE TO SPECIFIER **  Delete model not required.

      - Model Z7250 Failsafe with REX output as manufactured by SDC Security Door Controls.

      - Model Z7252 Failsecure with REX output as manufactured by SDC Security Door Controls.

** NOTE TO SPECIFIER **  Lever: Delete lever not required.

      - Lever: G -  Galaxy.

      - Lever: E -  Eclipse.

** NOTE TO SPECIFIER **  Finish Q standard. Delete finish not required.

      - Finish: Q - 626 Dull Chrome.

      - Finish: H - 613 Oil Rubbed Bronze.

      - Finish: C - 605 Bright Brass.

      - Finish: D - 606 Dull Brass.

      - Finish: P - 625 Bright Chrome.

** NOTE TO SPECIFIER **  Keying: Delete keying if not required.

      - Keying: SDC Conventional Cylinders (Schlage 'C' Keyway Standard).

** NOTE TO SPECIFIER **  For G1 and E1 levers. Delete cylinder not required.

        - Cylinder: 6PKA - 6 PIN Keyed alike.

        - Cylinder: 6PKD - 6 PIN Keyed different (standard).

** NOTE TO SPECIFIER **  Keying: Delete keying if not required.

      - Keying: SDC Interchangeable Core (Best 'A' Keyway Standard).

** NOTE TO SPECIFIER **  For G5 and E5 levers. Delete cylinder not required.

        - Cylinder: IC6P-KA - 6 pin I-Core, small format, keyed alike.

        - Cylinder: IC6P-KD - 6 pin I-Core, small format, keyed different.

        - Cylinder: IC7P-KD - 7 pin I-Core, small format, keyed different.

        - Cylinder: IC7P-KA - 7 pin I-Core, small format, keyed alike.

** NOTE TO SPECIFIER **  Strike: Delete strike not required.

      - Backset and Latch Bolt: B1 - 0.5625 inch (14 mm) throw with 2.75 inches (70 mm) backset.

      - Backset and Latch Bolt: B2 - 0.5625 inch (14 mm) throw with 2.375 inches (60 mm) backset.

** NOTE TO SPECIFIER **  Strike: Delete strike not required.

      - Strike: ST1 - ANSI A115.2, 1-1/4 inches by 4-7/8 inches (32 mm by 124 mm) (Standard).

      - Strike: ST2 - ANSI A115.2, 1-1/8 inches by 2-3/4 inches (29 mm by 70 mm) square corners.