  - ELECTRIFIED MORTISE LOCKSETS

    - ANSI Grade 1 Heavy Duty - Electrified Mortise Lockset:

      - Product: Z7800 Selectric Pro Electrified Mortise Lockset as manufactured by SDC Security Door Controls.

        - UL Listed for 3 hour fire rated doors.

        - Field selectable Failsafe or Failsecure.

        - Field selectable function: Locked outside only or both sides.

        - Field selectable 12/24 VDC.

        - Field selectable handing.

** NOTE TO SPECIFIER **  Delete function not required.

      - Function: Z7830 Failsafe, locked both sides.

      - Function: Z7832 Failsecure, locked both sides.

      - Function: Z7850 Failsafe, locked outside.

      - Function: Z7852 Failsecure, locked outside.

      - Function: Z7835 Proprietary design. Locked outside and/or inside. Outside failsecure, inside failsafe. Specify 12vdc or 24vdc

** NOTE TO SPECIFIER **  Handing: Delete handing not required.

      - Handing: L - Left hand.

      - Handing: LR - Left hand reversed.

      - Handing: R - Right hand.

      - Handing: RR - Right hand reversed.

** NOTE TO SPECIFIER **  Finish Q standard. Delete finish not required.

      - Finish: C - 605 Bright Brass.

      - Finish: D - 606 Dull Brass.

      - Finish: F - 611 Bright Bronze.

      - Finish: G - 612 Dull Bronze.

      - Finish: H - 613 Oil Rubbed Bronze.

      - Finish: P - 625 Bright Chrome.

      - Finish: Q - 626 Dull Chrome.

      - Finish: S - 629 Bright Stainless Steel.

      - Finish: U - 630 Dull Stainless.

** NOTE TO SPECIFIER **  Lever Trim: Delete trim not required.

      - Lever Trim: G - Galaxy with rose.

      - Lever Trim: GE - Galaxy with escutcheon.

      - Lever Trim: E - Eclipse with rose.

      - Lever Trim: EE - Eclipse with escutcheon.

      - Lever Trim: N - Nova with rose.

      - Lever Trim: NE - Nova with escutcheon.

      - Lever Trim: 07 - Schlage equivalent with SDC B rose.

      - Lever Trim: 07E - Schlage equivalent with SDC escutcheon.

      - Lever Trim: 12 - Schlage equivalent with SDC B rose.

      - Lever Trim: 12E - Schlage equivalent with SDC escutcheon.

** NOTE TO SPECIFIER **  Delete options not required.

      - Options/Suffix: R - Request-to-exit, SPDT.

      - Options/Suffix: D - Door Status, SPDT.

      - Options/Suffix: S - Latched and Locked, SPDT.

** NOTE TO SPECIFIER **  Keyed Cylinder: Delete cylinder not required.

      - Keyed Cylinder Optional: CYL-6KDQ - 6 pin, 1.125 inches (29 mm) mortise cylinder, keyed different, dull chrome.

      - Keyed Cylinder Optional: CYL-6KAQ - 6 pin, 1.125 inches (29 mm) mortise cylinder, keyed alike, dull chrome.